# МИНИАТЮРНЫЕ ТУМБЛЕРЫ

| Условное обозначение: Пример: MTS-102-А 1 MTS - тип 1 - количество контактных групп О2 - количество контактов в группе А - варианты рычажка 1 - варианты выводов | Технические характеристики: Номинальные значения: 10А 125 В / 6А 250 В пер.тока Контактное сопротивление: 20 МОм макс. Сопротивление изоляции: 100Mo мин.( 500В пост.тока) Электрическая прочность диэлектрика: 1500 в пер.тока за1 минуту Рабочая температура: от -55 С до 85 С Электрический ресурс: 20 000 циклов |
| --- | --- |

| MTS-101 ON-OFF SPST 2P |  |  |
| --- | --- | --- |

| MTS-102 ON-ON MTS-112 ON-(ON) MTS-103 ON-OFF-ON MTS-113 ON-OFF-(ON) MTS-123 (ON)-OFF-(ON) SPDT 3P |  |  |
| --- | --- | --- |

| MTS-201 ON-OFF DPST 4P |  |  |
| --- | --- | --- |

| MTS-202 ON-ON MTS-212 ON-(ON) MTS-203 ON-OFF-ON MTS-213 ON-OFF-(ON) MTS-223 (ON)-OFF-(ON) DPDT 6P |  |  |
| --- | --- | --- |

| MTS-101-A2 ON-OFF SPST 2P |  |  |
| --- | --- | --- |

| MTS-102-A2 ON-ON MTS-112-A2 ON-(ON) MTS-103-A2 ON-OFF-ON MTS-113 -A2 ON-OFF-(ON) MTS-123-A2 (ON)-OFF-(ON) SPDT 3P |  |  |
| --- | --- | --- |

| MTS-201-A2 ON-OFF DPST 4P |  |  |
| --- | --- | --- |

| MTS-202-A2 ON-ON MTS-212-A2 ON-(ON) MTS-203-A2 ON-OFF-ON MTS-213 -A2 ON-OFF-(ON) MTS-223-A2 (ON)-OFF-(ON) DPDT 6P |  |  |
| --- | --- | --- |

| MTS-102-A2T ON-ON MTS-112-A2T ON-(ON) MTS-103-A2T ON-OFF-ON MTS-113 -A2T ON-OFF-(ON) MTS-123-A2T (ON)-OFF-(ON) SPDT 3P |  |  |
| --- | --- | --- |

| MTS-202-A2T ON-ON MTS-212-A2T ON-(ON) MTS-203-A2T ON-OFF-ON MTS-213 -A2T ON-OFF-(ON) MTS-223-A2T (ON)-OFF-(ON) DPDT 6P |  |  |
| --- | --- | --- |

| MTS-102-C3 ON-ON MTS-112-C3 ON-(ON) MTS-103-C3 ON-OFF-ON MTS-113 -C3 ON-OFF-(ON) MTS-123-C3 (ON)-OFF-(ON) SPDT 3P |  |  |
| --- | --- | --- |

| MTS-202-C3 ON-ON MTS-212-C3 ON-(ON) MTS-203-C3 ON-OFF-ON MTS-213 -C3 ON-OFF-(ON) MTS-223-C3 (ON)-OFF-(ON) SPDT 6P |  |  |
| --- | --- | --- |

| MTS-102-C4 ON-ON MTS-112-C4 ON-(ON) MTS-103-C4 ON-OFF-ON MTS-113 -C4 ON-OFF-(ON) MTS-123-C4 (ON)-OFF-(ON) SPDT 3P |  |  |
| --- | --- | --- |

| MTS-202-C4 ON-ON MTS-212-C4 ON-(ON) MTS-203-C4 ON-OFF-ON MTS-213 -C4 ON-OFF-(ON) MTS-223-C4 (ON)-OFF-(ON) DPDT 6P |  |  |
| --- | --- | --- |

| MTS-101-E1 ON-OFF SPST 2P (PLASTIC FLATLEVER) |  |  |
| --- | --- | --- |

| MTS-102-E1 ON-ON MTS-112-E1 ON-(ON) MTS-103-E1 ON-OFF-ON MTS-113 -E1 ON-OFF-(ON) MTS-123-E1 (ON)-OFF-(ON) SPDT 3P (PLASTIC FLATLEVER) |  |  |
| --- | --- | --- |

| MTS-202-E1 ON-ON MTS-212-E1 ON-(ON) MTS-203-E1 ON-OFF-ON MTS-213 -E1 ON-OFF-(ON) MTS-223-E1 (ON)-OFF-(ON) DPDT 6P (PLASTIC FLATLEVER) |  |  |
| --- | --- | --- |

| MTS-101-F1 ON-OFF SPST 2P (METAL FLATLEVER) |  |  |
| --- | --- | --- |

| MTS-102-F1 ON-ON MTS-112-F1 ON-(ON) MTS-103-F1 ON-OFF-ON MTS-113 -F1 ON-OFF-(ON) MTS-123-F1 (ON)-OFF-(ON) SPDT 3P (METAL FLATLEVER) |  |  |
| --- | --- | --- |

| MTS-202-F1 ON-ON MTS-212-F1 ON-(ON) MTS-203-F1 ON-OFF-ON MTS-213 -F1 ON-OFF- (ON) MTS-223-F1 (ON)-OFF- (ON) DPDT 6P (METAL FLATLEVER) |  |  |
| --- | --- | --- |

| KNX-1 ON-ON KNX-103 ON-OFF-ON SPDT 3P |  |  |
| --- | --- | --- |

| KNX-2 ON-ON KNX-203 ON-OFF-ON DPDT 6P |  |  |
| --- | --- | --- |

| STM-101 ON-OFF SPST 2P |  |  |
| --- | --- | --- |

| STM-102 ON-ON STM-112 ON-(ON) STM-103 ON-OFF-ON STM-113 ON-OFF-(ON) STM-123 (ON)-OFF-(ON) SPDT 3P |  |  |
| --- | --- | --- |

| STM-202 ON-ON STM-212 ON-(ON) STM-203 ON-OFF-ON STM-213 ON-OFF-(ON) STM-223 (ON)-OFF-(ON) DPDT 6P |  |  |
| --- | --- | --- |

| MTS-101 ON-OFF SPST 2P |  |  |
| --- | --- | --- |

| STM-202-A3 ON-ON STM-212-A3 ON-(ON) STM-203-A3 ON-OFF-ON STM-213-A3 ON-OFF-(ON) STM-223-A3 (ON)-OFF-(ON) DPDT 6P |  |  |
| --- | --- | --- |